National Institute of Technology, Silchar

END-Semester (UG) Examination, Dec’2014

Subject code: EC 405, Subject: Optoelectronics and Optical Communication

Semester: 7th    Branch: ECE

Duration: Two Hours Total Marks: 50

Figure in the right margin indicated full marks for the question

| 1. (a) | In heterojunction laser much lower threshold current density is obtained as compared to homojunction laser - write the physical significance? | 3 |
| --- | --- | --- |
| (b) | Determine the threshold condition of gain in Fabry-Perot cavity. | 3 |
| (c) | Determine the power conversion efficiency of a GaAs which laser operates with a drive current much larger than . The cavity length is 250, the cavity loss co-efficient is 10 and the internal quantum efficiency is unity. | 5 |
| 2. | Show that in case of nN heterojunction the leakage current density is given by. | 6 |
| 3. | The following equations hold good for junction laser: Where is the excess carrier population,is the photon density,is stimulated emission,the loss of photons through the end walls , a relaxation term, I is the current and is the injection rate, A, B and C are appropriate parameters. Find i) Steady state solution ii) the threshold current for laser action. | 3+3 |
| 4.(a) | Draw the equivalent circuit of a PIN photodiode and write significance of each parameter. | 4 |
| (b) | What is NEP ? Determine NEP for photodiode. | 3 |
| 5.a) | What are the desirable characteristics of Optical Sources | 5 |
| b) | Why the practical spectral distribution of LED is different from the theoretical spectral distribution of LED | 5 |
| c) | Write a short note on Synchronous Networks | 5 |
| d) | Write a short note on Optical Transport Network. | 5 |

************************************************************************